| 3GPP TSG-T3 (USIM) Meeting #13 Tokyo, Japan, 21 - 24 February, 2000 | Tdoc T3-00 0057 |
| --- | --- |

Source: 	Gemplus

Title:	Proposed TSG-T WG3 work item for use of local link (RS232,  Bluetooth, USB, Irda) as a bearer for USIM Application Toolkit

Document for:	discussion and approval

__________________________________________________________________

We kindly ask that TSG-T  WG3 discuss and approve the attached work item.

# TSG WG3 - Work Item:
"Use of local link (RS232,  Bluetooth, USB, Irda) as a bearer for USAT"

## Justification and scope

The bearer independent protocol introduced recently (in SMG9) the possibility for the SIM the possibility to use several bearers to communicate with the network.

It is proposed to have a similar approach (bearer independent) in order to allow (U)SAT applications to communicate with local devices using the local connectivity capabilities of the terminal.

For application dedicated to third party equipment, the knowledge of the local environment (attached devices) is useful. So it is also proposed to have a way for the (U)SAT to get the local connection status, independent of the type of link.

Some application requires a very secure link. Security facilities offered by the bearer may be used, and if necessary an upper security layer could be defined. For example  an implementation of security mechanism specified in GSM 03.48 for bearer independent channel, could be considered.

This work item will result in appropriate Change Requests to 3G TS 31.111 and if necessary to the 3GPP equivalent of GSM 03.48.

## 3GPP work area

- TSG Terminals, WG3 (USIM)

## Linked work items

USIM Application Toolkit

## Service aspects

CRs will be proposed to S1 to include the service requirements for the feature in  3G TS 22.100 / 22.101 as appropriate

## MMI-aspects

- None.

## Charging aspects

- None.

## Security Aspects

- Security Requirements from TSG-SA WG3 shall be taken into account.

## Impacts

| - Affects: | - USIM | - ME | - NW | - Others |
| --- | --- | --- | --- | --- |
| - Yes | - X | - X |  |  |
| - No |  |  | - X |  |
| - Don't know |  |  |  |  |

## Expected Output and Timescales

| TSG adoption of WI | March 2000 |  |
| --- | --- | --- |
| Start of work | March 2000 |  |
| First complete draft | August 2000 | Version 1.0.0 |
| WG approval | October 2000 | Version 2.0.0 |
| TSG approval | December 2000 | Version 3.0.0 |

## Work item rapporteur

TBD

## Supporting companies

Gemplus, Bouygues telecom, Alcatel

## Primary responsible working party

- 3GPP TSG-T  WG3

## Secondary responsible working parties

None.

## Others

None.