| 3GPP T3 Meeting #14 Visby, Sweden, 24 - 26 May, 2000 | Tdoc T3-00 0188 |
| --- | --- |

| 3GPP TSG-T2 SWG1 London, UK March 28-29 2000 | T2x00025 |
| --- | --- |

To:	TSG T3

Source:	TSG T2 SWG1 (MExE)

Contact:	Lars Brenk (lars.brenk@dk.bosch.com)

Title:	Suggestion for a new mandatory HPLMNlen entry on the SIM cards

Document for:	Approval

### Introduction:

The MExE group is currently finalising the work for MExE Release 99 and MExE needs to be able to identify the operator, which issued a certain SIM card. For this purpose the IMSI is the only way of identifying the HPLMN, by taking the first 5 or 6 digits of the IMSI, as stated in section 2.2 in 23.003 v3.1.1. It is further mentioned in that section, that the two or three digit MNC topic, is out of that scope, and that further information can be found in GSM 03.22.

But GSM 03.22 only handles the case where a given PLMN, which the handset is registered on, and therefor knows if is five or six digits long, is compared against the IMSI HPLMN.

N2 stated in their answer to MExE’s question:

The question from T2 is: What is the proper way of extracting the right number of digits (five or six) from the IMSI, to find the HPLMN?

The answer from N2 is:

By examining the MCC digits, the necessary number of digits to be extracted from the MNC (two or three) is found.

This would mean, that a list of MCCs and the number of digits used to identify the PLMNs in that country, must be present in all MExE handsets. This list will be outdated at as soon as a new country (new MCC) is added to the GSM coverage family.

MExE would like to suggest, that a new mandatory field on the USIMs is dedicated to specifically identify the issuer (operator) of the USIM, by stating how many digits of the IMSI must be used to extract the PLMN.

The new entry has almost the same settings as the IMSI entry, since they are closely related. It is mandatory, under CHV1 protection and has the length of only 1 byte.

| 3GPP TSG-T2 /ETSI SMG4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | T2x00021 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| London, Britain, 28-29 March 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 11.11 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 8.0.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | For information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  | X | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Bosch Telecom Danmark |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 13.04.2000 |  |  |  |  |  |
| Subject: |  |  | New HPLMNlen entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | USIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | The ability to extract the right number of digits from the IMSI to determine the HPLMN (5 or 6 digits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 10.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | The addition is a necessity for the future work of the TWG T2 SWG1 (MExE) group. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 10.3.X	EFHPLMNlen (HPLMNlen)

This EF contains the length of the HPLMN, which is part of the International Mobile Subscriber Identity (IMSI).

| Identifier: '6FXX' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: 1 byte |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | length of HPLMN in the IMSI |  |  | M |  | 1 byte |

-	length of HPLMN in the IMSI

Contents:

The length indicator refers to the number of digits, required for extracting the HPLMN from the IMSI. (0x05 = 900/1800 band, 0x06=1900 band).