| 3GPP T3 Meeting #14 Visby, Sweden, 24 - 26 May, 2000 | Tdoc T3-000306 |
| --- | --- |

LIAISON STATEMENT

# From:	T3 (Contact Hubert Hélaine, Bouygues Telecom, +33 1 39 26 63 64)

# To:	N1

# CC:	new SMG9

# Title:	GPRS connection set up by SIM to create a channel between SIM and the end point destination

The USAT release 99 provides a feature to create a channel between SIM and a end point destination to exchange data using CSD or GPRS connection.

T3 has identified several problems in the current specification

- a mismatch between GPRS context address, access point name address, end point data destination address and  port number

- identification of the protocol layer provided by the ME to the SIM (SNDCP , UDP, TCP)

T3 asks N1 and any others relevant groups to revue the GPRS parameters provided by USIM in the “open channel command” for GPRS connection.

To open a channel on a GPRS bearer, the SIM provides the ME with the following parameters :

- Bearer description : Precedence class, Delay class, Reliability class, Peak throughput class, Mean throughput class, Packet data protocol type, Data compression, TCP/IP header Compression.

- Access point name : URL

- mobile local address : IP

- user login : text string

- user password : text string

in case of SIM uses TCP or UDP services provided by ME

- selected protocol service

- data destination address : URL or IP

- port number

You can find attached two CRs to the SIM toolkit specifications for your information.

We would be grateful if you could consider the above matter and provide us with the result of your discussions on this issue.