| 3GPP T3 Meeting #15 San Diego, USA, 16 - 18 August, 2000 | Tdoc T3-00 0344 |
| --- | --- |

Issues to be clarified for USIM conformance testing

Last Updated on :	21st July 2000

Author :	MCC Task 162

Summary :		This document contains a list of clarifications required to further the work by MCC task 162 to further the work on TS 31.122 (USIM conformance test specification). The issues listed in table A are questions or clarifications related to the core specifications, TS 31.101 and TS 31.102. The issues lists in table B are questions related to the GSM test specification GSM 11.17 or to the scope of testing required. The latest draft version of the TS 31.122 can be found on the 3GPP server under: http://www.3gpp.org/ftp/TSG_T/WG3_USIM/MCC-task162/TS-31122/

# Table A: Questions / Clarifications related to the core specification, TS 31.101 and TS 31.102

| No. | Reference | Description of the item in question | Actions to be taken / Status | Notes |
| --- | --- | --- | --- | --- |
| A1 | 3G 31.101 5 & 5.1 | ## Current consumption in the clock stop mode There is a convergence within the clause 5. At the beginning of the clause it says ; “When the UICC is in the clock stop mode, the current consumption shall not exceed 100uA at 25 centigrade” On the contrary, the clause 5.1.1 says ; “ If clock mode is enabled, then the current mode shall also not exceed 200uA while the clock is stopped”. Which value shall we follow here? | Need clarification. | 06/13 TK |
| A2 | 3G 31.101 9.3 | ## Security Environment How do we change the current Security Environment to another? A command like Manage Security Environment command is not defined in 3G 31.101 or 3G 31.102. | Need Clarification. | 06/14 SK |
| A3 | 3G 31.101 | ### Definition for idle state There is no definition for the word “idle state” although it is used in the 3G 31.101. It seems that it is better to include the definition used in the GSM11.11 if it is the same. (See below) There are two states for the SIM while the power supply is on: ->The SIM is in operating state when it executes a command. This state also includes transmission from and to the ME. ->The SIM is in idle state at any other time. It shall retain all pertinent data during this state. | Need Clarification. | 06/14 TK |
| A4 | 3G 31.101 11.1.2 | ## STATUS Command What does it mean by 'currently selected application'? If DFPHONEBOOK under DFUSIM is selected by FID, what is the response data when the STATUS command with P2 = 1 is sent to the UICC, given that DFPHONEBOOK has a DF name? What would be the response like if DFPHONEBOOK doesn't have the DF name? If DFPHONEBOOK under DFUSIM is selected by DF name, what is the response data when the STATUS command with P2 = 1 is sent to the UICC? | Need Clarification. | 06/23 |
| A5 | 3G 31.101 11.1.1.4.3 | ## File Descriptor The value of the length of the File Descriptor is supposed to be '01' and '05', where the present document specifies the value as '01' and '07'. | Need clarification. | 06/23 TK |
| A6 | 3G 31.101 11.1.7 | SEARCH RECORD command Two different interpretations exist. [Interpretation A] A record is recognized as 'found' by the UICC when it has the specified search pattern within the record, or when it has the specified search pattern from the specified offset to the end of the record in case offset is presented. [Interpretation B] A record is recognized as 'found' by the UICC when the record is starting from the specified search pattern, or when it has the specified search pattern starting from the presented offset in case offset is presented. | Need clarification. Refer to an explanatory document “ Two different interpretations for SEARCH RECORD command” for this question. (see T3-000345) | 06/23 TK |
| A7 | 3G 31.101 11.1.7 | SEARCH RECORD command Present document does not specify the meaning of P1='00'. Is following understanding correct? - P1='00' means "current record". - P1='00' is applicable to all the search modes. - Any record number can be used in enhanced forward search from next record mode and enhanced backward search from previous record mode. | Need clarification. | 07/05 TK |
| A8 | 3G 31.101 11.1.7 | SEARCH RECORD command After an error in the SEARCH RECORD command, is the record pointer remained unchanged? | Need clarification. | 07/05 |

# Table B: Questions related to the GSM test specification GSM 11.17 or to the scope of testing required

| No. | Reference | Description of the item in question | Actions to be taken / Status | Notes |
| --- | --- | --- | --- | --- |
| B1 | GSM11.17 6.1.3 | The corresponding part in the GSM11.17 does not make sense. What is the purpose of the test? In addition, the CR1 and CR3 are missing. | Need clarification | 06/13 TK |
| B2 | 3G 31.101 14 | Do we need to incorporate the tests for application independent protocol to 3G 31.122? It seems that the tests for this part should be incorporated to 3G 31.120. | Need Clarification. | 06/13 |
| B3 | 3G 31.102 5 | Do we need to incorporate the tests for USIM application protocol to 3G 31.122? It seems that the tests for this part should be incorporated to 3G 31.121. | Need Clarification. | 06/13 |