| 3GPP TSG-T3 (USIM) Meeting #8 Bonn, Germany, August 23 - 25, 1999 | Tdoc T3-99224 |
| --- | --- |

3GPP TSG-SA WG 2		TDoc S2-99633

26-30 July 1999		Rev of Tdoc  S2-99584

New Jersey, USA

Liaison statement

To:				3GPP TSGs S3, T3

Copy:	3GPP TSG N2

Source:	3GPP TSG S2

Title:	Answer to the liaison on Interoperation between UMTS and GSM

S2 has considered the liaison from S3 on their initial thought on GSM/UMTS interoperation for security related issues.

S2 supports the view expressed in the referenced liaison statement on the feasibility of re-authentication at inter-system handover.

For the location of the conversion function:

Based on the assumption that the set of vectors can be provided either independently or together within the MAP operation between an UMTS HLR and any nodes (MSC or SGSN) requesting these vectors –i.e. an upgraded node will receive both sets and a non upgraded node shall only receive the necessary vector- S2 is of the opinion that it would be preferable that the HLR shall contain the conversion functions. Note that this is a first analysis, and that does not prevent further analysis of the different solutions.

Further information is needed to decide the location of the conversion function on the mobile side.

If the working assumption made by S2 is incorrect then S2 would like to know what is the exact intention of S3 on this matter and is welcoming the idea of the joint session proposed by S3 to discuss with other involved Working Group the full issue. S2 Relevant material (e.g. for inter-system handover procedure) will be presented during the joint meeting. Unfortunately the date proposed is colliding with the date of the next S2 meeting but it will be ensured that delegate from S2 shall attend this meeting in order S2 and S3 to be able to complete the issue in due time for inclusion in the release 99.