TSG-T3 (USIM) meeting #11		T3-99420

Sophia Antipolis 8th-10th December, 1999		(replaces T3-99412)

LIAISON STATEMENT

Title:	Capability configuration parameters

Source:	TSG-T3

To: 	TSG-N1

TSG-T3 (USIM) kindly asks TSG-N1 to provide some information about the configuration and capability parameters to be stored into USIM under EFCCP at the light that new bearer capabilities have been (or will be)  introduced in 3G.

In particular, TSG-T3 would like to know if the current record length and coding of the EFCCP (see below) defined in GSM 11.11 and GSM 04.08 are suitable to store the relevant information for 3G environment defined in 3G TS 24.008.

A fast response would be highly appreciated since the information is needed in order to finalise 3G TS 31.102 for Release 99.

### EFCCP (Capability configuration parameters)

This EF contains parameters of required network and bearer capabilities and ME configurations associated with a call established using an abbreviated dialling number, a fixed dialling number, an MSISDN, a last number dialled, a service dialling number or a barred dialling number.

| Identifier: '6F3D' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 14 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE CHV1 INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to 10 | Bearer capability information element |  |  | M |  | 10 bytes |
| 11 to 14 | Bytes reserved - see below |  |  | M |  | 4 bytes |

-	Bearer capability information element

Contents and Coding:

- see GSM 04.08 [15].  The Information Element Identity (IEI) shall be excluded. i.e. the first byte of the EFCCP record shall be Length of the bearer capability contents.

-	Bytes 11-14 shall be set to 'FF' and shall not be interpreted by the ME.